REPORT OF DESIGNING A NETWORK STRUCTURE

CREATING A NETWORK TOPOLOY

WITH

PROVIDING SECURITY

OWNER

Trust-in-depth

DATE: 30/05/2025

Table of Contents

1.Contents 2

2.Network Topology Overview4

3.Hostname Configurations 5

HQ Side5

Branch Side5

Internet (Remote, ISP, Unrelated) 5

4.VLAN Configuration6

1.HQ SIDE6

Inter-VLAN Routing6

Sub-Interfaces6

Native VLAN7

Management VLAN8

2.BRANCH SIDE9

Inter-VLAN Routing9

Sub-Interfaces9

Native VLAN10

Management VLAN11

3. Explanation of command VLANS12

Vtp(Rute-on-a-stick) 12

Native13

Management13

5.IP Addressing14

1.DHCP Configuration14

HQ DHCP Router14

ISP_DHCP SERVER15

2.Manual IP 16

Manually Configured PC’S 16

Manually Configured Servers17

Manually Configured Router Interfaces18

6.Routing (EIGRP) 20

Configuration Per Router20

Summarization and Wildcard Masks21

7.NAT Configuration22

HQ Side22

Branch Side23

8.Server Configurations25

Web Server25

DNS Server25

FTP Server25

Mail Server26

DHCP Server26

9.Security Measures27

ACL Rules and Lists27

SSH and Password Policies29

MAC Address Port Security32

10.WAN Configuration (PPP) 34

PPP Encapsulation34

IP Address Assignments34

11.VPN Configuration35

ISAKMP & IPSec Setup35

ACL for VPN Traffic36

Crypto Map Binding36

12.Etherchannel37

13.STP(Spanning Tree Protocol) 38

14.The Problems I Faced39

TOPOLOGY

Hostname Configurations

| hq_router | core_switch | data_switch | hr_switch | it_switch | finance_switch |
| --- | --- | --- | --- | --- | --- |
| Switch#en Switch#conf t Switch(config)#host HQ HQ(config)# | Switch#en Switch#conf t Switch(config)#host CORE CORE(config)# | Switch>en Switch#conf t Switch(config)#host DATA DATA(config)# | Switch>en Switch#conf t Switch(config)#host HR HR(config)# | Switch>en Switch#conf t Switch(config)#host IT IT(config)# | Switch>en Switch#conf t Switch(config)#host FNC FNC(config)# |

hq_side

| branch_router | branch_core_switch | sale_switch | research_switch | costumer_sup_switch |
| --- | --- | --- | --- | --- |
| Router>en Router#conf t Router(config)#host BRANCH BRANCH(config)# | Switch>en Switch#conf t Switch(config)#host BR_CORE BR_CORE(config)# | Switch>en Switch#conf t Switch(config)#host SALE SALE(config)# | Switch>en Switch#conf t Switch(config)#host R&D R&D(config)# | Switch>en Switch#conf t Switch(config)#host CS CS(config)# |

branch_side


Internet_side(REMOTE)

| DEFAULT(router) | REMOTE(router) | Rm_sw(switch) |
| --- | --- | --- |
| Router>en Router#conf t Router(config)#host DEFAULT DEFAULT (config)# | Router>en Router#conf t Router(config)#host REMOTE REMOTE (config)# | Switch>en Switch#conf t Switch(config)#host rm_sw rm_sw(config)# |

Internet_side(ISP)

| ISP_RT(Router) | ISP_SW(Switch) |
| --- | --- |
| Router>en Router#conf t Router(config)#host ISP_RT ISP_RT(config)# | Switch>en Switch#conf t Switch(config)#host ISP_SW ISP_SW(config)# |

Internet_side(UNRELATED)

| UNRELATED(Router) | unr_sw(switch) |
| --- | --- |
| Router>en Router#conf t Router(config)#host UNRELATED UNRELATED(config)# | Switch>en Switch#conf t Switch(config)#host unr_sw unr_sw(config)# |

What do those commands of hostname configuration do?

Router>en                       #en means “enable”, it turn device enable mode to be full connected

Router#conf t                 # it is “configure terminal”.  We can change settings

Router(config)#host -------                #Host is “hostname”. With this we give a name to our device

UNRELATED(config)#                         #We can see the changes -Router is converted to UNRELATED

Summary:

- We are using it to give a name each our devices

VLANS

HQ SIDE

-- Inter-VLAN Routing(Router-on-a-Stick)--

Sub-interfaces

| HQ (config-if)#int g0/0.10 HQ (config-subif)#encapsulation dot1Q 10 HQconfig-subif)#ip address 192.168.10.1 255.255.255.224 HQ (config-subif)#int g0/0.20 HQ(config-subif)#encapsulation dot1Q 20 HQ (config-subif)#ip address 192.168.10.33 255.255.255.224 HQ (config-subif)#int g0/0.30 HQ (config-subif)#encapsulation dot1Q 30 HQ (config-subif)#ip address 192.168.10.65 255.255.255.224 HQ (config-subif)#int g0/0.40 HQ (config-subif)#encapsulation dot1Q 40 HQ (config-subif)#ip address 192.168.10.97 255.255.255.224 |  |
| --- | --- |
| -----No shut down HQ(config)#int g0/0 HQ(config-if)#no sh |  |

What those commands above(sub-interface) do?

HQ (config-subif)#int g0/0.20                      #making sub interface in this there is g0/0 interface and we split that interfaces as a sub-interface

HQ(config-subif)#encapsulation dot1Q 20                                            #we are tagging the data to demonstrate that which data is going where

HQ (config-subif)#ip address 192.168.10.33 255.255.255.224         #giving IP address to sub-interface

HQ(config)#int g0/0                                                                                  #going real interface

HQ(config-if)#no sh                                                                                  #prentent to be down

*WE do this because we want to vlans to can communicate outside device so there will be each vlan gateway address

VTP Configuration (VLAN Trunking Protocol)

Step 1:

CREATE VTP NETWORK 
-Create a domain for vtp

-Set the core_sw as a vtp server and give a password

-Created vlans

Step 2:

DEFINE RELATIONS( SERVER-CLIENT)

On each switch excluded core;

-Create their vlans( it is important

-Give same domain name for each

-Make them a client

- Give them a password

| CORE Switch | The structure |
| --- | --- |
| CORE>en CORE#conf t CORE(config)#vtp domain VTP_NETWORK CORE(config)#vtp mode server Device mode already VTP SERVER. CORE(config)#vtp password core CORE(config)#vlan 10 CORE(config-vlan)#name DATA CORE(config-vlan)#vlan 20 CORE(config-vlan)#name HR CORE(config-vlan)#vlan 30 CORE(config-vlan)#name IT CORE(config-vlan)#vlan 40 CORE(config-vlan)#name FNC |  |

| DATA_SWITCH | HR_SWITCH | IT_SWITCH | FNC_SWITCH |
| --- | --- | --- | --- |
| DATA#conf t DATA(config)#vtp domain VTP_NETWORK DATA(config)#vtp mode client DATA(config)#vtp password core | HR#en HR#conf t HR(config)#vtp domain VTP_NETWORK HR(config)#vtp mode client HR(config)#vtp password core | IT>en IT#conf t IT(config)#vtp domain VTP_NETWORK IT(config)#vtp mode client IT(config)#vtp password core | FNC>en FNC#conf t FNC(config)#vtp domain VTP_NETWORK FNC(config)#vtp mode client FNC(config)#vtp password core |

Step 3:

DETERMINE PORT WHICH WILL HAS TRUNK, WHICH WILL HAS ACCESS MODE

--Organise the ports either trunk or access

-Configure each end_device interface connected to switch as access mode

-make device connect to vlan

| DATA_SWITCH | HR_SWITCH | IT_SWITCH | FNC_SWITCH |  |
| --- | --- | --- | --- | --- |
| DATA>en DATA#conf t DATA(config)#int range f0/1-4 DATA(config-if-range)#switchport mode access DATA(config-if-range)#switchport access vlan 10 DATA(config-if-range)#ex DATA(config)#int f0/10 DATA(config-if)#switchport mode trunk | HR#en HR#conf t HR(config)#int range f0/1-3 HR(config-if-range)#switchport mode access HR(config-if-range)#switchport access vlan 20 HR(config-if-range)#ex HR(config)#int f0/10 HR(config-if)#switchport mode trunk | IT>en IT#conf t IT(config)#int range f0/1-4 IT(config-if-range)#switchport mode access IT(config-if-range)#switchport access vlan 30 IT(config-if-range)#ex IT(config)#int f0/10 IT(config-if)#switchport mode trunk | FNC#en FNC#conf t FNC(config)#int range f0/1-4 FNC(config-if-range)#switchport mode access FNC(config-if-range)#switchport access vlan 40 FNC(config-if-range)#ex FNC(config)#int f0/10 FNC(config-if)#switchport mode trunk | CORE>en CORE#conf t CORE(config)#int range f0/2-5 CORE(config-if-range)#switchport mode trunk |

Native Vlan:

The native VLAN is used for untagged traffic on a trunk port

| core | data | hr | it | fnc |
| --- | --- | --- | --- | --- |
| core>en core#conf t core(config)#vlan 99 core(config-vlan)#name Native_VLAN core(config-vlan)#int range f0/10-13 core(config-if-range)#switchport trunk native vlan 99 core(config-if-range)#int g0/2 core(config-if)#sw trunk native vlan 99 | DATA(config)#int f0/10 DATA(config-if)#switchport access vlan 99 DATA(config-if)#switchport trunk native vlan 99 | HR#conf t HR(config)#int f0/10 HR(config-if)#switchport access vlan 99 HR(config-if)#switchport trunk native vlan 99 | IT#conf t IT(config)#int f0/10 IT(config-if)#sw ac vlan 99 IT(config-if)#sw tr nat vlan 99 | FNC#conf t FNC(config)#int f0/10 FNC(config-if)#sw acc vlan 99 FNC(config-if)#sw tr nat vlan 99 |

Management VLAN

The management VLAN is used to manage the switch (SSH, Telnet, web, etc.)

| core | data | hr | it | fnc | hq |
| --- | --- | --- | --- | --- | --- |
| core#conf t core(config)#vlan 15 core(config-vlan)#name MANAGEMENT core(config-vlan)#int vlan 15 core(config-if)#ip address 192.168.10.130 255.255.255.248 core(config)#ip default-gateway 192.168.10.129 | DATA(config)#int f0/10 DATA(config-if)#sw access vlan 15 DATA(config)#ex DATA(config)#int vlan 15 DATA(config-if)#ip address 192.168.10.131 255.255.255.248 DATA(config-if)#no sh DATA(config-if)#ex DATA(config)#ip default-gateway 192.168.10.129 | HR#conf t HR(config)#int f0/10 HR(config-if)#sw acc vlan 15 HR(config-if)#int vlan 15 HR(config-if)#ip address 192.168.10.132 255.255.255.248 HR(config-if)#no sh HR(config-if)#ex HR(config)#ip default-gateway 192.168.10.129 | IT#conf t IT(config)#int f0/10 IT(config-if)#sw acc vlan 15 IT(config-if)#int vlan 15 IT(config-if)#ip add 192.168.10.133 255.255.255.248 IT(config-if)#no sh IT(config-if)#ex IT(config)#ip default-gateway 192.168.10.129 | FNC#conf t FNC(config)#int f0/10 FNC(config-if)#sw acc vlan 15 FNC(config-if)#int vlan 15 FNC(config-if)#ip add 192.168.10.134 255.255.255.248 FNC(config-if)#no sh FNC(config-if)#ex FNC(config)#ip default-gateway 192.168.10.129 | HQ(config)#int g0/0.15 HQ(config-subif)#encapsulation dot1Q 15 HQ(config-subif)#ip address 192.168.10.129 255.255.255.248 Note:router olduğundan vlan gibi bir yapıdan çok sub inerface şeklinde yapılandırıldı. |

BRANCH SIDE

-- Inter-VLAN Routing(Router-on-a-Stick)--

Sub-interfaces

| BRANCH(config)#int g0/0.10 BRANCH(config-subif)#encapsulation dot1Q 10 BRANCH(config-subif)#ip address 192.168.5.1 255.255.255.224 BRANCH(config-subif)#int g0/0.20 BRANCH(config-subif)#encapsulation dot1Q 20 BRANCH(config-subif)#ip address 192.168.5.33 255.255.255.224 BRANCH(config-subif)#int g0/0.30 BRANCH(config-subif)#encapsulation dot1Q 30 BRANCH(config-subif)#ip address 192.168.5.65 255.255.255.224 |  |
| --- | --- |
| ---for no sut down BRANCH(config)#int g0/0 BRANCH(config-if)#no sh |  |

VTP Configuration (VLAN Trunking Protocol)

| BR_CORE Switch | The structure |
| --- | --- |
| BR_CORE>en BR_CORE #conf t BR_CORE (config)#vtp domain BR_NETWORK BR_CORE (config)#vtp mode server Device mode already VTP SERVER. BR_CORE (config)#vtp password br_core BR_CORE (config)#vlan 10 BR_CORE (config-vlan)#name SALE BR_CORE (config-vlan)#vlan 20 BR_CORE (config-vlan)#name R&D BR_CORE (config-vlan)#vlan 30 BR_CORE (config-vlan)#name CS |  |

| SALE(Switch) | R&D(Switch) | CS (Switch) |
| --- | --- | --- |
| SALE#en SALE#conf t SALE (config)#vtp domain VTP_NETWORK SALE (config)#vtp mode client SALE (config)#vtp password br_core | R&D#en R&D #conf t R&D (config)#vtp domain VTP_NETWORK R&D (config)#vtp mode client R&D (config)#vtp password br_core | CS>en CS#conf t CS(config)#vtp domain VTP_NETWORK CS(config)#vtp mode client CS(config)#vtp password br_core |

| Br_core | SALE(Switch) | R&D(Switch) | CS (Switch) |
| --- | --- | --- | --- |
| BR_CORE(config)#int range f0/20-22 BR_CORE(config-if-range)#switchport mode trunk BR_CORE(config)#int g0/2 BR_CORE(config-if)#switchport mode trunk | SALE(config)#int range f0/1-10 SALE(config-if-range)#switchport mode access SALE(config-if-range)#switchport access vlan 10 SALE(config-if-range)#ex SALE(config)#int f0/20 SALE(config-if)#switchport mode trunk | R&D(config)#int range f0/1-8 R&D(config-if-range)#switchport mode access R&D(config-if-range)#sw access vlan 20 R&D(config-if-range)#ex R&D(config)#int f0/20 R&D(config-if)#switchport mode trunk | CS(config)#int range f0/1-8 CS(config-if-range)#switchport mode access CS(config-if-range)#switchport access vlan 30 CS(config-if-range)#ex CS(config)#int f0/20 CS(config-if)#switchport mode trunk |

Interface Configuration(Trunk-Acces)

Native Vlan:

| br_core | sale | r&d | cs |
| --- | --- | --- | --- |
| BR_CORE#conf t Enter configuration BR_CORE(config)#vlan 99 BR_CORE(config-vlan)#name Native_Vlan BR_CORE(config-vlan)#int range f0/20-22 BR_CORE(config-if-range)#switchport trunk native vlan 99 BR_CORE(config-if-range)#int g0/2 BR_CORE (config-if)#sw trunk native vlan 99 | SALE#conf t SALE(config)#int f0/20 SALE(config-if)#sw acc vlan 99 SALE(config-if)#sw tr nat vlan 99 | R&D#conf t R&D(config)#int f0/20 R&D(config-if)#sw acc vlan 99 R&D(config-if)#sw tr nat vlan 99 | CS#conf t CS(config)#int f0/20 CS(config-if)#sw acc vlan 99 CS(config-if)#sw tr nat vlan 99 |

| We can see the result of : br_core, Hr, R&D Beside them for the other native the structure of vlans will be like that |  |  |
| --- | --- | --- |

Management VLAN

| Br_core | sale | R&d | cs | branch |
| --- | --- | --- | --- | --- |
| BR_CORE#conf t BR_CORE(config)#vlan 15 BR_CORE(config-vlan)#name MANAGEMENT BR_CORE(config-vlan)#int vlan 15 BR_CORE(config-if)#ip address 192.168.5.130 255.255.255.248 BR_CORE(config-if)#ex BR_CORE(config)#ip default-gateway 192.168.10.129 | SALE#conf t SALE(config)#int f0/20 SALE(config-if)#sw ac vlan 15 SALE(config-if)#int vlan 15 SALE(config-if)#ip address 192.168.5.131 255.255.255.248 SALE(config-if)#ex SALE(config)#ip default-gateway 192.168.5.129 | R&D#conf t R&D(config)#int f0/20 R&D(config-if)#sw ac vlan 15 R&D(config-if)#int vlan 15 R&D(config-if)#ip address 192.168.5.132 255.255.255.248 R&D(config-if)#ex R&D(config)#ip default-gateway 192.168.5.129 | CS#conf t CS(config)#int f0/20 CS(config-if)#sw ac vlan 15 CS(config-if)#int vlan 15 CS(config-if)# CS(config-if)#ip address 192.168.5.133 255.255.255.248 CS(config-if)#ex CS(config)#ip default-gateway 192.168.5.129 | BRANCH#conf t BRANCH(config)#int g0/0.15 BRANCH(config-subif)#encapsulation dot1Q 15 BRANCH(config-subif)#ip address 192.168.5.129 255.255.255.248 |

After Configuring vlans:

|  | After configuration like beside, T defines Trunk ports A defines Access ports |
| --- | --- |

Explanation of command VLANS

1-VTP (route-on-a stick)

ALL 3 Step commands. What do those command do?

---Creating vlan network and determining server and client relationship---

CORE>en                # Access to device

CORE#conf t           #give permission to change setting which is terminal

CORE(config)#vtp domain VTP_NETWORK          # We are creating a domain network which is named VTP_NETWORK

CORE(config)#vtp mode server

# We are saying that device will be server so clients cannot create their own vlan but server can do that and share it with clients

Device mode already VTP SERVER.                        #We get this because they are server as a default

CORE(config)#vtp password core                            #We have already made our network and now we are giving a password for it

CORE(config)#vlan 10                                             #We are creating vlans

CORE(config-vlan)#name DATA                             # naming the vlan

DATA#conf t

DATA(config)#vtp domain VTP_NETWORK           # We wanna get vlan name from VTP_SERVER so we shoul be same network

DATA(config)#vtp mode client

#That command says we are client so we cannot create any vlan just can get them from VTR_SERVER

DATA(config)#vtp password core   #For that network we should have same password to reach network

---Defining which interface will be in trunk or access mode---

DATA(config)#int range f0/1-4                                                              #to conf a range of interface at the same time

DATA(config-if-range)#switchport mode access                                #it is default mode to connect a device

DATA(config-if-range)#switchport access vlan 10                             #it provides to connect to vlan 10 for all interface in range

DATA(config-if-range)#ex                                                                       #it is “exit” to exit conf-if-range

DATA(config)#int f0/10                                                                          #we can go and configure f0/10 interface via this command

DATA(config-if)#switchport mode trunk                                            #we turned mode to trunk to make access ports like a just one port

Sum:

- Firstly, we are making a vlan network and then on switch side define who will be trunk and who will be access, also on router side we are making sub-interfaces to give gateway to vlans

2-Native Vlan

core>en                                                                             #to reach device

core#conf t                                                                       #to make some changes on setting

core(config)#vlan 99                                                       # creating vlan

core(config-vlan)#name Native_VLAN                         # naming the vlan

core(config-vlan)#int range f0/10-13                            # to go some interface range

core(config-if-range)#switchport trunk native vlan 99                   #defining vlan 99 on range, as a native vlan

core(config-if-range)#int g0/2                                         #going to interface g0/2

core(config-if)#sw trunk native vlan 99                          #creating native vlan on interface g0/2

Sum:

- If we remember, at the begining we tagged traffic with dot1Q, in case of untagged traffic we will be using native vlan to route the untagged packet.

3-Management Vlan

HR#conf t                                            # being able to configuring

HR(config)#int f0/10                         # to go interface

HR(config-if)#sw acc vlan 15

# it is actually “switchport access vlan 15”.  In vtp system that means the server created vlan 15 and now client try to access it.

HR(config-if)#int vlan 15                                                                 #we accessed vlan 15 ant going interface of vlan 15

HR(config-if)#ip address 192.168.10.132 255.255.255.248     #giving management vlan ip address

HR(config-if)#no sh                                                                         #to prevent it from being down

HR(config-if)#ex                                                                              #exiting from the last mode

HR(config)#ip default-gateway 192.168.10.129                       #determining gateway ip address to reach outside devices

Sum:

- we are using management vlan to manage switches like sh,telnet, etc.

- We was split our IP mask /24 to two part: 1.one is 192.168.10.0/25 and another one is 192.168.128.0/25 so that we created them because that in any case we can use it. It that state, we use 2. part IPs to give vlan

IP ADDRESSING

DHCP CONFIGURATION

We have 2 system to serve us ips:

1-HQ (Router) =it provides ips to HQ(headquarter)

2-ISP_DHCP(Server) = provides ips to all network under an ISP and of course for branch office.

HQ ROUTER SİDE

-Creating pools

HQ (config)#ip dhcp pool vlan10 HQ (dhcp-config)#network 192.168.10.0 255.255.255.224 HQ(dhcp-config)#default-router 192.168.10.1 HQ (dhcp-config)#dns-server 192.168.10.8 HQ (dhcp-config)#ip dhcp pool vlan20 HQ (dhcp-config)#network 192.168.10.32 255.255.255.224 HQ (dhcp-config)#default-router 192.168.10.33 HQ (dhcp-config)#dns-server 192.168.10.8 HQ (dhcp-config)#ip dhcp pool vlan30 HQ (dhcp-config)#network 192.168.10.64 255.255.255.224 HQ (dhcp-config)#default-router 192.168.10.65 HQ (dhcp-config)#dns-server 192.168.10.8 HQ (dhcp-config)#ip dhcp pool vlan40 HQ (dhcp-config)#network 192.168.10.96 255.255.255.224 HQ (dhcp-config)#default-router 192.168.10.97 HQ (dhcp-config)#dns-server 192.168.10.8

-excluding gateway addresses

HQ (config)#ip dhcp excluded-address 192.168.10.1 HQ (config)#ip dhcp excluded-address 192.168.10.33 HQ (config)#ip dhcp excluded-address 192.168.10.65 HQ (config)#ip dhcp excluded-address 192.168.10.97

Explanation of commands:

HQ (config)#ip dhcp pool vlan10                                                       #creating pool and naming

HQ (dhcp-config)#network 192.168.10.0 255.255.255.224         #giving a network address with subnetmask

HQ(dhcp-config)#default-router 192.168.10.1                                 #giving IP address of gateway

HQ (dhcp-config)#dns-server 192.168.10.8                                      #giving DNS server IP address

HQ (config)#ip dhcp excluded-address 192.168.10.1                      #Excluding IP address so not to give that IP any device

SUM:

- Before everything we created pool, which has a scale of IP address in it, to share IP address to other devices.

- The reason not to include gateway IP of each vlans is that we don’t want to give that IP address other devices and make gateway IP stabile to block some conflict.

- And also give IP address of DNS to can reverse IP address when it is needed

DHCP SERVER SIDE (ISP_DHCP)

branch

BRANCH#conf t BRANCH(config)#int g0/0.10 BRANCH(config-subif)#ip helper-address 192.168.1.2 BRANCH(config-subif)#int g0/0.20 BRANCH(config-subif)#ip helper-address 192.168.1.2 BRANCH(config-subif)#int g0/0.30 BRANCH(config-subif)#ip helper-address 192.168.1.2

Meaning of them?
BRANCH(config)#int g0/0.10

#Going to sun-interface

BRANCH(config-subif)#ip helper-address 192.168.1.2

#Introducing IP address of DHCP as a helper because We decided use DHCP to get IP address not to give IP address manually

Sum:

- we give each sub-interface ip of helper-address to get IP address and can communicate with other devices

MANUALLY

I am doing some part manually due to being able to write restriction like some ACL list or rules and also giving each servers an IP address manually because that pc’s should be able to reach them every time and prevent some conflict. Thus, servers should have stable IP addresses.

Manually Configured PC’S
hr_1:

it_1:

Fnc_1:

Sale_1:

Sale_2:

R&D:

CS:

Unrelated:

Rm1(remote_user):

Manually Configured Servers

WEB_SERVER:

FTP_SERVER:

MAIL_SERVER:

DNS_SERVER:

MANUALLY CONFIGURED ROUTER INTERFACES

-Explanation of command above:

HQ>en                                                   #to connect device

HQ#conf t	           	                    #make configuration

HQ(config)#int s0/1/0                        #go to interface

HQ(config-if)#ip address 100.100.100.1   255.255.255.252       #give IP address that interface

HQ(config-if)#no sh                             #to open the port

*It is same for each interface

| HQ_s0/1/0 | ISP_router_s0/1/0 | ISP_router_s0/1/1 | branch_s0/1/1 | ISP_router_g0/1 |
| --- | --- | --- | --- | --- |
| HQ>en HQ#conf t HQ(config)#int s0/1/0 HQ(config-if)#ip address 100.100.100.1 255.255.255.252 HQ(config-if)#no sh | ISP_RT#conf t ISP_RT(config)#int s0/1/0 ISP_RT(config-if)#ip address 100.100.100.2 255.255.255.252 ISP_RT(config-if)#no sh | ISP_RT#conf t ISP_RT(config)#int s0/1/1 ISP_RT(config-if)#ip address 100.100.100.5 255.255.255.252 ISP_RT(config-if)#no sh | BRANCH#conf t BRANCH (config)#int s0/1/1 BRANCH (config-if)#ip address 100.100.100.6 255.255.255.252 BRANCH (config-if)#no sh | ISP_RT(config)#int g0/1 ISP_RT(config-if)#ip address 192.168.1.1 255.255.255.252 ISP_RT(config-if)#no sh |

| HQ_s0/0/0 | HQ_s0/1/1 | Remote_s0/1/0 | Remote_g0/1 |
| --- | --- | --- | --- |
| HQ>en HQ#conf t HQ(config)#int s0/0/0 HQ(config-if)#ip address 100.100.100.17 255.255.255.252 HQ(config-if)#no sh | HQ>en HQ#conf t HQ(config)#int s0/1/1 HQ(config-if)#ip address 100.100.100.9 255.255.255.252 HQ(config-if)#no sh | REMOTE#conf t REMOTE (config)#int s0/1/0 REMOTE (config-if)#ip address 100.100.100.14 255.255.255.252 REMOTE (config-if)#no sh | REMOTE#conf t REMOTE (config)#int g0/1 REMOTE (config-if)#ip address 192.168.10.137 255.255.255.248 REMOTE (config-if)#no sh |

| Default_s0/1/0 | Default_s0/1/1 | Unrelated_s0/0/0 | Unrelated_g0/1 |
| --- | --- | --- | --- |
| DEFAULT>en DEFAULT #conf t DEFAULT (config)#int s0/1/0 DEFAULT (config-if)#ip address 100.100.100.13 255.255.255.252 DEFAULT (config-if)#no sh | DEFAULT>en DEFAULT #conf t DEFAULT (config)#int s0/1/1 DEFAULT (config-if)#ip address 100.100.100.10 255.255.255.252 DEFAULT (config-if)#no sh | UNRELATED#conf t UNRELATED (config)#int s0/0/0 UNRELATED (config-if)#ip address 100.100.100.18 255.255.255.252 UNRELATED (config-if)#no sh | UNRELATED#conf t UNRELATED (config)#int g0/1 UNRELATED (config-if)#ip address 200.200.200.1 255.255.255.252 UNRELATED (config-if)#no sh |

EIGRP

ROUTING(EIGRP)

| branch | Isp_rt | hq | remote | default | unrelated |
| --- | --- | --- | --- | --- | --- |
| BRANCH>en BRANCH#conf t BRANCH (config)#router eigrp 10 BRANCH (config-router)#eigrp router-id 1.1.1.1 BRANCH (config-router)#network 192.168.5.0 0.0.0.31 BRANCH (config-router)#network 192.168.5.32 0.0.0.31 BRANCH (config-router)#network 192.168.5.64 0.0.0.31 BRANCH (config-router)#network 100.100.100.4 0.0.0.3 BRANCH (config-router)#no auto-summary | ISP_RT>en ISP_RT#conf t ISP_RT(config)#route eigrp 10 ISP_RT(config-router)#eigrp router-id 2.2.2.2 ISP_RT(config-router)#network 0.0.0.0 0.0.0.0 (internet) | HQ>en HQ#conf t HQ(config)#route eigrp 10 HQ(config-router)#eigrp router-id 3.3.3.3 HQ(config-router)#network 192.168.10.0 0.0.0.31 HQ(config-router)#network 192.168.10.32 0.0.0.31 HQ(config-router)#network 192.168.10.64 0.0.0.31 HQ(config-router)#network 192.168.10.96 0.0.0.31 HQ(config-router)#network 100.100.100.0 0.0.0.3 HQ(config-router)#network 192.168.10.128 0.0.0.127(I added that to make other device can make ssh connection) HQ(config-router)#no auto-summary | REMOTE#conf t REMOTE(config)#router eigrp 10 REMOTE(config-router)#eigrp router-id 4.4.4.4 REMOTE(config-router)#network 0.0.0.0 0.0.0.0 (internet) | DEFAULT#conf t DEFAULT(config)#router eigrp 10 DEFAULT(config-router)#eigrp router-id 5.5.5.5 DEFAULT(config-router)#network 0.0.0.0 0.0.0.0 (internet) | DEFAULT#conf t DEFAULT(config)#router eigrp 10 DEFAULT(config-router)#eigrp router-id 5.5.5.5 DEFAULT(config-router)#network 0.0.0.0 0.0.0.0 (internet) |

What are the routing commands?

BRANCH>en                                                                                           #to access device

BRANCH#conf t                                                                                      #to set some sonfiguration  t is terminal so “configure terminal”

BRANCH (config)#router eigrp 10                                                       #determinin type of routing protocol and Autonomous System (AS) number

BRANCH (config-router)#eigrp router-id 1.1.1.1                               #giving a router-id

BRANCH (config-router)#network 192.168.5.0 0.0.0.31              # This tells the router to enable EIGRP on interfaces matching 192.168.5.0/27

BRANCH (config-router)#network 192.168.5.32 0.0.0.31               #Same above it also says there is a network to neighbour router

BRANCH (config-router)#network 192.168.5.64 0.0.0.31                #Making some advertisements to share that info

BRANCH (config-router)#network 100.100.100.4 0.0.0.3                #same as the last of 3

BRANCH (config-router)#no auto-summary                                       #prevent sytem from rounding curtain network

ISP_RT(config)#route eigrp 10                                          # specify type of routing algorithm and AS number

ISP_RT(config-router)#eigrp router-id 2.2.2.2                #as a formality, it just help us what is happing and who is who

ISP_RT(config-router)#network 0.0.0.0 0.0.0.0              #it says I will sent every message (ads) from everwhere (so assuming like it is internet)

(internet)

Sum:

- To begin with, we specifying router protocol and autonomous system number, If we haven’t used same AS number, then they wouldn’t be able to make a connection. Therefore. I supposed there is just one AS and it is 10. Then I remarked every network in interfaces on each router (I do that for Branch and HQ). And also, I assumed there was internet between Branch and HQ so I remarked all interface network as 0.0.0.0 and wildcard mask 0.0.0.0 so that mains route every network.

NAT CONFIGURATIONS

HQ_SIDE

| HQ#conf t HQ(config)#int g0/0.10 HQ(config-subif)#ip nat inside HQ(config-subif)#int g0/0.20 HQ(config-subif)#ip nat inside HQ(config-subif)#int g0/0.30 HQ(config-subif)#ip nat inside HQ(config-subif)#int g0/0.40 HQ(config-subif)#ip nat inside HQ(config-subif)#int s0/1/0 HQ(config-if)#ip nat outside HQ(config-if)#ex HQ(config)#access-list 110 deny ip 192.168.10.0 0.0.0.127 192.168.5.0 0.0.0.127 HQ(config)#access-list 110 permit ip 192.168.10.0 0.0.0.127 any HQ(config)#ip nat pool NATPOOL 1.1.1.5 1.1.1.13 netmask 255.255.255.0 HQ(config)#ip nat inside source list 110 pool NATPOOL |  |
| --- | --- |

To show that it works or not?

| First ping from hr_1 to sale_1 Second ping from hr_1 to ISP_DHCP | After first ping we cannot see any translation because we denied packets that go to 192.168.5.0 from being NAT’ed. We can see via second ping, the IP translation and of course because of NAT we send ping but the ping cannot return due to IP changing |
| --- | --- |

HQ#conf t                                                                             #to configure terminal of HQ

HQ(config)#int g0/0.10                                                      #going to sub interface of g0/0.10

HQ(config-subif)#ip nat inside                                          #indicating that device inside will be NAT’ed

HQ(config-subif)#int g0/0.20                                            # going to sub-interface of g0/0.20

HQ(config-subif)#ip nat inside                                          # saying: launch NAT inside

HQ(config-subif)#int g0/0.30                                            # going to sub-interface of g0/0.30

HQ(config-subif)#ip nat inside                                          # writing this, make IP of devices in that interface network change

HQ(config-subif)#int g0/0.40                                            # going to sub-interface of g0/0.40

HQ(config-subif)#ip nat inside                                          #Inside will be NAT’ed

HQ(config-subif)#int s0/1/0                                              # to go interface s0/1/0

HQ(config-if)#ip nat outside                                             #That means the NAT’ted IP go out via that interface

HQ(config-if)#ex                                                                  #exit from conf_if

----Creating an acl for NAT----

HQ(config)#access-list 110 deny ip 192.168.10.0 0.0.0.127 192.168.5.0 0.0.0.127

#create an acl named 110, deny a packet from HQ network to branch

*(we will be using this for NAT ,so actual meaning of it that deny any packet from HQ to be NAT so like it is not allowed to change its IP)

HQ(config)#access-list 110 permit ip 192.168.10.0 0.0.0.127 any

#give direction that permit a packet from HQ network to everyone to same acl number(110)

HQ(config)#ip nat pool NATPOOL 1.1.1.5 1.1.1.13 netmask 255.255.255.0

#creating NAT pool named NATPOOL 
#get an IP range( a range from 5-13) from ISP
#we are using that when we send packet that packet will go through that interface IP.

HQ(config)#ip nat inside source list 110 pool NATPOOL

#We are saying that use acl 110 and when you translate ip give an IP them from pool

(we could do with interface and PAT system like I made it for BRANCH, I ended up with NATPOOL for scalability)

BRANCH _SİDE NAT

| BRANCH#conf t BRANCH(config-if)#int g0/0.10 BRANCH(config-subif)#ip nat inside BRANCH(config-subif)#int g0/0.20 BRANCH(config-subif)#ip nat inside BRANCH(config-subif)#int g0/0.30 BRANCH(config-subif)#ip nat inside BRANCH(config-if)#int s0/1/1 BRANCH(config-if)#ip nat outside BRANCH(config-if)#ex BRANCH(config)#access-list 110 deny ip 192.168.5.0 0.0.0.127 192.168.10.0 0.0.0.127 BRANCH(config)#access-list 110 permit ip 192.168.5.0 0.0.0.127 any BRANCH(config)#ip nat pool NATPOOL 1.1.1.14 1.1.1.23 netmask 255.255.255.0 BRANCH(config)#ip nat inside source list 110 interface s0/1/1 overload |  |
| --- | --- |

To show that it works or not?

| First ping from cs_1 to it_3 Second ping from cs_1 to unrelated | There was made overload PAT (Port Address Translation) , we used same inside global as our s0/1/1 interface IP Also we couldn’t see first ping NAT ,because we write ACL about avoiding NAT when packet goes to HQ |
| --- | --- |

----Creating an acl for NAT----

BRANCH(config)#access-list 110 deny ip 192.168.5.0 0.0.0.127 192.168.10.0 0.0.0.127

#create an acl named 110, deny a packet from BRANCH network to HQ

*(we will be using this for NAT ,so actual meaning of it that deny any packet from BRANCH to be NAT so like it is not allowed to change its IP)

BRANCH(config)#access-list 110 deny ip 192.168.5.0 0.0.0.127 host 192.168.1.2

#We prevent BRANCH network translation for ISP_DHCP to get their IP addresses without any problem.

BRANCH(config)#access-list 110 permit ip 192.168.5.0 0.0.0.127 any

#give direction that permit a packet from BRANCH network to everyone to same acl number(110)

BRANCH(config)#ip nat pool NATPOOL 1.1.1.14 1.1.1.23 netmask 255.255.255.0

#creating NAT pool named NATPOOL 
#get an IP range( a range from 14-23) from ISP
#we are using that when we send packet that packet will go through that interface IP.

BRANCH(config)#ip nat inside source list 110 interface s0/1/1 overload

#We are saying that use acl 110 and when you translate ip give an IP them interface and use PAT, I mean make sending packet on ports and use s0/0/0 interface (I settle on that one.)

Sum:

- First of all, we indicate each interface which will be inside and which will be outside, then we create an ACL, actually that part crucial because that when we send our message from HQ and NAT’ed that packet IP then when we face another NAT’ted network we couldn’t be communicate. Because router in between them won’t be able to understand which IP is belong to whom. Prevent this we use NAT. And for sure, We can indicate when IP in HQ will be NAT’ed wit help of the ACL. When we write ACL deny that will be meaning of deny that IP address from being Nat’ed to specific network then, the Packet will go that network without any change on its IP address. I permit other network to be denied so when we go internet no one can see that packet actual Ip address. Also, about acl, in first rule I wrote deny translate ip from hq then for second, giving permission to any address. There is no conflict. What we write first it is accepted by ACL. ACL apply the rules row by row below. To put in a nut shell, we made that conf both BRANCH and HQ side to be able to communicate and not to show real IP to internet.

SERVERS

1.WEB

-We turned HTTP off because it includes lots of vulnerability.

-When someone connect web server they get a page like beside.

2.DNS

-Image right side indicates that when we write domain name a browser and when it returns an answer that means that domain name was resolved by a DNS.

3.FTTP

The right side of page there is an image of ftp connection and its result

4. MAİL

-For mail server, We configured the sytem like givina a domain name and add users.
-Then, created mail address for most of PC’s

-The last step was sending mail and approve whether it was sent

5. DHCP

Server

SECURITY OF THE NETWORK

ACL (ACCESS CONTROL LIST)

HQ

| List 110 | Explanation of commands applied for NAT |
| --- | --- |
| HQ#conf t HQ(config)#ip access-list extended 110 HQ(config-ext-nacl)#10 deny ip 192.168.10.0 0.0.0.127 192.168.5.0 0.0.0.127 HQ(config-ext-nacl)#20 permit ip 192.168.10.0 0.0.0.127 any -- applied to -- HQ(config)#ip nat inside source list 110 pool NATPOOL | ip access-list extended 110 = we are creating a list named 110 and it is extended so we can make restriction on ports as well rule 10 – deny packet from IP translation of HQ Network 10.0/25 when packet goes to BRANCH Network rule20 –give permission to change ip address when a HQ packet go anywhere |

| List 120 | Explanation of commands * Sub-interface |
| --- | --- |
| HQ#conf t HQ(config)#ip access-list extended 120 HQ(config-ext-nacl)#10 permit tcp host 192.168.10.98 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#20 permit tcp host 192.168.10.34 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#30 permit tcp host 192.168.10.66 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#40 permit tcp host 192.168.10.138 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#50 permit tcp host 192.168.5.34 host 192.168.10.6 eq ftp HQ(config)#60 permit tcp host 192.168.5.2 host 192.168.10.6 eq ftp HQ(config)#70 deny tcp any host 192.168.10.6 eq ftp HQ(config)#80 permit icmp any any HQ(config)#90 permit ip any any --applied to -- HQ(config-ext-nacl)#ex HQ(config)#int g0/0.10 HQ(config-subif)#ip ac HQ(config-subif)#ip access-group 120 in HQ(config-subif)#int g0/0.20 HQ(config-subif)#ip access-group 120 in HQ(config-subif)#int g0/0.30 HQ(config-subif)#ip access-group 120 in HQ(config-subif)#int g0/0.40 HQ(config-subif)#ip access-group 120 in | IP access-list extended 120 = we are creating a list named 120 and it is extended so we can make restriction on ports as well Rule 10, just 98 PC can reach FTP server Rule 20, just 10.34 PC can reach FTP server Rule 30, just 66 PC can reach FTP server Rule 40, just 138 PC can reach FTP server Rule 50, just 5.34 PC can reach FTP server Rule 60, just 5.2 PC can reach FTP server Rule 70, no one can reach FTP server Rule 80, everyone can ping each other Rule 90, any IP can reach any IP *Acl was implemented to sub-interfaces ip access-group 120 in #that means put list 120 the inbound of interface so checking the packet come from inside not going out |

| List 130 | Explanation of the commands *ISP_DHCP serve DNS future |
| --- | --- |
| HQ#conf t HQ(config)#ip access-list extended 130 HQ(config-ext-nacl)#10 permit tcp host 192.168.5.2 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#20 permit tcp host 192.168.5.34 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#30 permit tcp host 192.168.5.66 host 192.168.10.6 eq ftp HQ(config-ext-nacl)#40 deny tcp any host 192.168.10.6 eq ftp HQ(config-ext-nacl)#50 permit tcp 192.168.5.0 0.0.0.255 host 192.168.10.7 eq smtp HQ(config-ext-nacl)#60 permit udp host 192.168.1.2 any HQ(config-ext-nacl)#70 permit udp any host 192.168.1.2 eq domain HQ(config-ext-nacl)#80 permit tcp 192.168.5.0 0.0.0.255 host 192.168.10.5 eq 443 HQ(config-ext-nacl)#90 permit eigrp any any HQ(config-ext-nacl)#100 deny icmp any 192.168.10.0 0.0.0.31 HQ(config-ext-nacl)#110 permit icmp any any -applied to— HQ(config)#int s0/1/0 HQ(config-if)# ip access-group 110 in | IP access-list extended 130 = we are creating a list named 130 and it is extended so we can make restriction on ports as well Rule 10, just 5.2 PC can reach FTP server via 21 port Rule 20, just 5.34 PC can reach FTP server via 21 port Rule 30, just 5.66 PC can reach FTP server via 21 port Rule 40, block every request for ftp to FTP server via 21 port Rule 50, full BRANCH Network can send mail to MAIL server via 25 port Rule 60, ISP_DHCP can send udp packet everywhere Rule 70, every packet from 53 port is allowed to reach ISP_DHCP Rule 80, only BRANCH Network can reach WEB server via 443 port Rule 90, permit every eigrp packet (hello packets) Rule 100, block everone from ping to HQ Network Rule 110, everyone can ping each other *we applied that rules to inbound of interface s0/1/0 |

| List 150 | Explanation of the commands *s0/0/0 |
| --- | --- |
| HQ#conf t HQ(config)#ip access-list extended 150 HQ(config-ext-nacl)#10 permit tcp any host 192.168.10.5 eq 443 HQ(config-ext-nacl)#20 permit tcp host 192.168.10.5 any eq 443 HQ(config-ext-nacl)#30 deny ip 192.168.10.0 0.0.0.255 any HQ(config-ext-nacl)#40 deny ip any 192.168.10.0 0.0.0.255 HQ(config-ext-nacl)#50 permit ip any any -applied to— HQ(config)#int s0/0/0 HQ(config-if)# ip access-group 150 in | IP access-list extended 150 = we are creating a list named 150 and it is extended so we can make restriction on ports as well Rule 10, everyone can reach WEB server via 443 port Rule 20, WEB server can reach everyone via 443 port Rule 30, no one in HQ Network can reach other PC’s IP Rule 40, everyone was blocked to reach HQ Network Rule 50, everyone can ping each other *we applied that rules to inbound of interface s0/0/0 |

| List 170 | Explanation of the commands *used for VPN |
| --- | --- |
| HQ#conf t HQ(config)#ip access-list extended 170 HQ(config-ext-nacl)#10 permit ip 192.168.10.0 0.0.0.255 192.168.10.136 0.0.0.7 --applied to— HQ(config-crypto-map)#match address 170 ** crypto map VPN_MAP **s0/1/1 | IP access-list extended 150 = we are creating a list named 150 and it is extended so we can make restriction Rule 10, make encryption on packet from HQ Network to REMOTE Network *That rule is applied to s0/1/1 for determining our MAP |

BRANCH

| List 110 | Explanation of commands applied for NAT |
| --- | --- |
| BRANCH#conf t BRANCH(config)#ip access-list extended 110 BRANCH(config-ext-nacl)#10 deny ip 192.168.5.0 0.0.0.127 192.168.10.0 0.0.0.127 BRANCH(config-ext-nacl)#20 deny ip 192.168.5.0 0.0.0.127 host 192.168.1.2 BRANCH(config-ext-nacl)#30 permit ip 192.168.5.0 0.0.0.127 any -- applied to -- BRANCH(config)#ip nat inside source list 110 interface s0/1/1 overload | ip access-list extended 110 = we are creating a list named 110 and it is extended so we can make restriction on ports rule 10 – deny packet from IP translation of BRANCH Network 5.0/25 when packet goes to HQ Network rule 20 –deny BRANCH packets IP translation when it goes to ISP_DHCP (is maden because of that PC’s getting IP address from ISP_DHCP) rule 30 –give permission to change ip address when a BRANCH packet go anywhere *The rule list applied for NAT in s0/1/1 interface |

| List 120 | Explanation of commands applied for NAT |
| --- | --- |
| BRANCH#conf t BRANCH(config)#ip access-list extended 120 BRANCH(config-ext-nacl)#10 permit eigrp any any BRANCH(config-ext-nacl)#20 permit ip 192.168.10.0 0.0.0.255 any BRANCH(config-ext-nacl)#30 permit ip host 192.168.1.2 any BRANCH(config-ext-nacl)#40 deny ip any any -applied to— BRANCH(config)#int s0/1/1 BRANCH (config-if)# ip access-group 120 in | ip access-list extended 110 = we are creating a list named 110 and it is extended so we can make restriction on ports rule 10 – every eigrp packet is allowed to go everywhere rule 20 –HQ Network can reach BRANCH Network rule 30 –ISP_DHCP is allowed to connect BRANCH Network rule 40 – every PC is denied to connect any PC *access-group(list 120) is applied on inbound of interface s0/1/0 |

SUM:

With those rules we specified that certain IP and port such as 53,443,21,22 only connect certain IP or ports of devices

SSH-Enable_Console_Password

(Enable_Console)Password

| hq | core | branch | br_core |
| --- | --- | --- | --- |
| HQ>en HQ#conf t HQ(config)#enable secret hq HQ(config)#line console 0 HQ(config-line)#password hq HQ(config-line)#login HQ(config-line)#exec-timeout 5 0 HQ(config-line)#logging synchronous HQ(config-line)#exit HQ(config)#ip domain-name office.local HQ(config)#crypto key generate rsa --How many bits in the modulus [512]: 1024 HQ(config)#username admin privilege 15 secret hq HQ(config)#line vty 0 4 HQ(config-line)#login local HQ(config-line)#exec-timeout 5 0 HQ(config-line)#transport input ssh HQ(config-line)#exit HQ(config)#security passwords min-length 10 HQ(config)#username user password hq % Password too short - must be at least 10 characters. Password not configured. HQ(config)#username user password hq_user_hq HQ(config)#login block-for 180 attempts 3 within 60 HQ(config)#banner motd "Unauthorized access is not allowed" | core>en core#conf t core(config)#enable secret core core(config)#line console 0 core(config-line)#password core core(config-line)#login core(config-line)#exec-timeout 5 0 core(config-line)#logging synchronous core(config)#ip domain-name office.local core(config)#crypto key generate rsa ---How many bits in the modulus [512]: 1024 core(config)#line vty 0 4 core(config-line)#login local core(config-line)#exec-timeout 5 0 core(config-line)#transport input ssh core(config-line)#exit core(config)#username admin privilege 15 secret core core(config)#banner motd "Unauthorized access is not allowed" core(config)#login block-for 180 attempts 3 within 60 | BRANCH>en BRANCH#conf t BRANCH(config)#security passwords min-length 10 BRANCH(config)#enable secret branch_center_switch BRANCH(config)#login block-for 180 attempts 3 within 60 BRANCH(config)#ip domain-name branch_affice.local BRANCH(config)#crypto key generate rsa How many bits in the modulus [512]: 1024 BRANCH(config)#username admin privilege 15 secret branch BRANCH(config)#line vty 0 4 BRANCH(config-line)#login local BRANCH(config-line)#exec-timeout 5 0 BRANCH(config-line)#transport input ssh BRANCH(config-line)#exit BRANCH(config)#line console 0 BRANCH(config-line)#password branch_center_switch BRANCH(config-line)#login BRANCH(config-line)#exec-timeout 5 0 BRANCH(config-line)#logging synchronous BRANCH(config-line)#exit BRANCH(config)#banner motd "Unauthorized access is not allowed" | BR_CORE#conf t BR_CORE(config)#enable secret br_core BR_CORE(config)#username admin privilege 15 secret br_core BR_CORE(config)#ip domain-name branch_office.local BR_CORE(config)#crypto key generate rsa ---How many bits in the modulus [512]: 1024 BR_CORE(config)#line vty 0 4 BR_CORE(config-line)#login local BR_CORE(config-line)#exec-timeout 5 0 BR_CORE(config-line)#transport input ssh BR_CORE(config-line)#exit BR_CORE(config)#line console 0 BR_CORE(config-line)#password br_core BR_CORE(config-line)#exec-timeout 5 0 BR_CORE(config-line)#logging synchronous BR_CORE(config)#banner motd "Unauthorized access is not allowed." |

SSH connection to core and hq was succeded via it_1                                             SSH connection to br_core and branch was succeded via cs_1

*In the images, it_1 connected to HQ via ssh connection, at the end it was able to reach. We can see looking at the last line

| data | hr | it | fnc |
| --- | --- | --- | --- |
| DATA#conf t DATA(config)#enable secret data DATA(config)#line consol 0 DATA(config-line)#password data DATA(config-line)#login DATA(config-line)#exec-timeout 5 0 DATA(config-line)#logging synchronous DATA(config-line)#exit DATA(config)#service password-encryption | HR#conf t HR(config)#enable secret hr HR(config)#line console 0 HR(config-line)#password hr HR(config-line)#login HR(config-line)#exec-timeout 5 0 HR(config-line)#logging synchronous HR(config-line)#exit HR(config)#service password-encryption | IT#conf t IT(config)#enable secret it IT(config)#line console 0 IT(config-line)#password it IT(config-line)#login IT(config-line)#exec-timeout 5 0 IT(config-line)#logging synchronous IT(config-line)#exit IT(config)#service password-encryption | FNC#conf t FNC(config)#enable secret fnc FNC(config)#line con 0 FNC(config-line)#password fnc FNC(config-line)#login FNC(config-line)#exec-timeout 5 0 FNC(config-line)#logging synchronous FNC(config-line)#exit FNC(config)#service password-encryption |

| sale | r&d | cs |
| --- | --- | --- |
| SALE#conf t SALE(config)#enable secret sale SALE(config)#line con 0 SALE(config-line)#password sale SALE(config-line)#login SALE(config-line)#exec-timeout 5 0 SALE(config-line)#logging synchronous SALE(config-line)#exit SALE(config)#service password-encryption | R&D#conf t R&D(config)#enable secret r&d R&D(config)#line con 0 R&D(config-line)#password r&d R&D(config-line)#login R&D(config-line)#exec-timeout 5 0 R&D(config-line)#logging synchronous R&D(config-line)#exit R&D(config)#service password-encryption | CS#conf t CS(config)#enable secret cs CS(config)#line con 0 CS(config-line)#password cs CS(config-line)#login CS(config-line)#exec-timeout 5 0 CS(config-line)#logging synchronous CS(config-line)#exit CS(config)#service password-encryption |

What are the commands?

HQ>en                                                                 #connect Switch

HQ#conf t                                                           #makine changes on terminal

HQ(config)#enable secret hq                          #to connect or make SW enable we create password(hq) and make it secret so it is not plain text

HQ(config)#line console 0                                #to go console 0

HQ(config-line)#password hq                         #to create a password for console 0

HQ(config-line)#login                                       # it means when we see a line is like “password: “, after try login

HQ(config-line)#exec-timeout 5 0                  # When there is a inactivity in 5 minutes 0 second then session will be closed

HQ(config-line)#logging synchronous            #When I write commands,system messages wont interrupt me

HQ(config-line)#exit                                          #exiting from line console 0

HQ(config)#ip domain-name office.local      #giving domain name for ssh like creating new envorioument for ssh connection

HQ(config)#crypto key generate rsa                           #using crypto and make a key in rsa

--How many bits in the modulus [512]: 1024            #that bey will be 1024 bits

HQ(config)#username admin privilege 15 secret hq

#creating user who has the highest authority( 15) and creating encrypted password(hq)

HQ(config)#line vty 0 4                      #at the same that only 5 connection is allowed for ssh connection to HQ under the network (office.local)

HQ(config-line)#login local                #just that enviourement devices ( office.local) is permitted

HQ(config-line)#exec-timeout 5 0                              # When there is a inactivity in 5 minutes 0 second then session will be closed

HQ(config-line)#transport input ssh                          #only permission to ssh no other connection type is allowed such as telnet.

HQ(config-line)#exit                                                      # #exiting from line vty

HQ(config)#security passwords min-length 10

#it used to specify some rule when password created. It says password should be at least 10 characters

HQ(config)#username user password hq                            # when we create a new user and password. The error raised ad say:

% Password too short - must be at least 10 characters. Password not configured.

HQ(config)#username user password hq_user_hq           #we created that user with longer password

HQ(config)#login block-for 180 attempts 3 within 60

#in 180 second inactivity or more then 3 times wrong password attempts it will close session and after 3 incorrect attempts, there will be a 60 second waiting time.

HQ(config)#banner motd "Unauthorized access is not allowed"

#Putting a massage of the day , it will be seen after login to warn unauthorized user.

DATA#conf t                                                              #configure terminal

DATA(config)#enable secret data                          #when want to connect device there will be encrypted password (data) with (password 7)

DATA(config)#line consol 0                                     #go to console

DATA(config-line)#password data                          #set password for console

DATA(config-line)#login                                            #that password request will be shown on login line

DATA(config-line)#exec-timeout 5 0                       #after 5 min inactivity, session will be closed

DATA(config-line)#logging synchronous                #not to be interrupted by system logs

DATA(config-line)#exit                                               #login out from line

DATA(config)#service password-encryption           #make every password in system encrypted

SUM:

- We created user and its password in secure condition such as using encryption and also using security system. We create password for console, enable, ssh connection. For ssh we create a local domain network. Besides, we blocked any telnet which not secure connection.
*Putting duration in system 
* Giving specific attempts

*Determining length of password

as not to be exposed easily.

MAC-Violation

| data | hr | it | fnc |
| --- | --- | --- | --- |
| DATA#conf t DATA(config)#int range f0/1-4 DATA(config-if-range)#sw port-security DATA(config-if-range)#switchport port-security mac-address sticky DATA(config-if-range)#switchport port-security maximum 1 range)#switchport port-security violation shutdown | HR>en HR#conf t HR(config)#int range f0/1-3 HR(config-if-range)#sw port-security mac-address sticky HR(config-if-range)#switchport port-security maximum 1 HR(config-if-range)#switchport port-security violation shutdown | IT#en IT#conf t IT(config)#int range f0/1-4 IT(config-if-range)#sw port-security IT(config-if-range)#switchport port-security mac-address sticky IT(config-if-range)#switchport port-security maximum 1 IT(config-if-range)#switchport port-security violation shutdown | FNC>en FNC#conf t FNC(config)#int range f0/1-4 FNC(config-if-range)#sw port-security FNC(config-if-range)#sw port-security mac-address sticky FNC(config-if-range)#sw port-security maximum 1 FNC(config-if-range)#sw port-security violation shutdown |

After conf ports turn enabled and they look like:

| sale | b&d | cs |
| --- | --- | --- |
| SALE>en SALE#conf t SALE(config)#int range f0/1-10 SALE(config-if-range)#sw port-security SALE(config-if-range)#sw port-security mac-address sticky SALE(config-if-range)#sw port-security maximum 1 SALE(config-if-range)#sw port-security violation shutdown | R&D#conf t R&D(config)#int range f0/1-8 R&D(config-if-range)#sw port-security R&D(config-if-range)#sw port-security mac-address sticky R&D(config-if-range)#sw port-security maximum 1 R&D(config-if-range)#sw port-security violation shutdown | CS>en CS#conf t CS(config)#int range f0/1-8 CS(config-if-range)#sw port-security CS(config-if-range)#sw port-security mac-address sticky CS(config-if-range)#sw port-security maximum 1 CS(config-if-range)#sw port-security violation shutdown |

After process:

What are the commands?

IT#en                                                          #connect IT

IT#conf t                                                     #make changes on IT configuration

IT(config)#int range f0/1-4                      #going to certain interfaces simultaniously

IT(config-if-range)#switchport port-security mac-address sticky            # it enables sticky MAC address learning on a switch port.

IT(config-if-range)#switchport port-security maximum 1                         #just one device can connect that interface

IT(config-if-range)#switchport port-security violation shutdown            #if there is a violation then shut interface down

SUM:
*WE are using mac violation because we matching each interface just for one PC, if there is an pyshical attack, it make us triggered and we will prevent it because after MAC changing we close the connection to new MAC address.

WAN(PPP)
Links and IP addresses

Conf of WAN

| DEFAULT(s0/1/1) | DEFAULT (s0/1/0) |
| --- | --- |
| DEFAULT>en DEFAULT #conf t DEFAULT (config)#int s0/1/1 DEFAULT (config-if)#ip address 100.100.100.10 255.255.255.252 DEFAULT (config-if)#no sh DEFAULT (config-if)#encapsulation ppp | DEFAULT (config)#ex DEFAULT (config)#int s0/1/0 DEFAULT (config-if)#ip address 100.100.100.13 255.255.255.252 DEFAULT (config-if)#no sh DEFAULT(config-if)#encapsulation ppp |

| REMOTE(s0/1/0) | REMOTE(g0/1) |
| --- | --- |
| REMOTE>en REMOTE #conf t REMOTE (config)#int s0/1/0 REMOTE (config-if)#ip address 100.100.100.14 255.255.255.252 REMOTE (config-if)#no sh REMOTE (config-if)#encapsulation ppp | REMOTE>en REMOTE #conf t REMOTE (config)#int g0/1 REMOTE (config-if)#ip address 192.168.10.137 255.255.255.248 REMOTE (config-if)#no sh |

| BRANCH(s0/1/1) | UNRELATED |
| --- | --- |
| BRANCH>en BRANCH(config)#int s0/1/1 BRANCH(config-if)#ip address 100.100.100.6 255.255.255.252 BRANCH(config-if)#no sh BRANCH(config-if)#encapsulation ppp | BRANCH>en BRANCH(config)#int s0/0/0 BRANCH(config-if)#ip address 100.100.100.18 255.255.255.252 BRANCH(config-if)#no sh BRANCH(config-if)#encapsulation ppp |

| HQ (s0/1/0) | HQ(s0/1/1) | HQ(s0/0/0) |
| --- | --- | --- |
| HQ>Password: HQ#conf t HQ(config)#int s0/1/0 HQ(config-if)#ip address 100.100.100.1 255.255.255.252 HQ(config-if)#no sh HQ(config-if)#encapsulation ppp | HQ(config-if)#ex HQ(config)#int s0/1/1 HQ(config-if)#ip address 100.100.100.9 255.255.255.252 HQ(config-if)#no sh HQ(config-if)#encapsulation ppp | HQ(config-if)#ex HQ(config)#int s0/0/0 HQ(config-if)#ip address 100.100.100.17 255.255.255.252 HQ(config-if)#no sh HQ(config-if)#encapsulation ppp |

What the commands do?

REMOTE>en                                      #enable device

REMOTE #conf t                                 #we can configure terminal

REMOTE (config)#int s0/1/0            # going to interface s0/1/0

REMOTE (config-if)#ip address 100.100.100.14 255.255.255.252   #giving ip address

REMOTE (config-if)#no sh    #not allowed to shut interface down

REMOTE (config-if)#encapsulation ppp

#it is point-to-point protocol and it provides authentication on serial interfaces

| ISP_RT (s0/1/0) | ISP(s0/1/1) | ISP(g0/1) |
| --- | --- | --- |
| ISP_RT>en ISP_RT#conf t ISP_RT(config)#int s0/1/0 ISP_RT(config-if)#ip address 100.100.100.2 255.255.255.252 ISP_RT(config-if)#no sh ISP_RT(config-if)#encapsulation ppp | ISP_RT(config)#int s0/1/1 ISP_RT(config-if)#ip address 100.100.100.5 255.255.255.252 ISP_RT(config)#no sh ISP_RT(config-if)#encapsulation ppp | ISP_RT(config)#int g0/1 ISP_RT(config-if)#ip address 192.169.1.1 255.255.255.252 ISP_RT(config)#no sh |

SUM:

- we give IP address manually on serial interfaces and encapsulates packet into a PPP frame to providing security in Layer 2

VPN CONFIGURATION

Checking for configuring security files like (k9):

Even though we have that file, due to not having security license the vpn couldn’t implemented, so :

we ended up getting security packet:

HQ(config)#license boot module c1900 technology-package securityk9 REMOTE(config)#license boot module c1900 technology-package securityk9

| REMOTE | HQ |
| --- | --- |
| REMOTE(config)#crypto isakmp policy 10 REMOTE(config-isakmp)#encr aes REMOTE(config-isakmp)#hash sha REMOTE(config-isakmp)#authentication pre-share REMOTE(config-isakmp)#group 2 REMOTE(config-isakmp)#lifetime 86400 REMOTE(config)#crypto isakmp key VPN_KEY address 100.100.100.9 REMOTE(config)#crypto ipsec transform-set VPN_SET esp-aes esp-sha-hmac REMOTE(config)#access-list 170 permit ip 192.168.10.136 0.0.0.7 192.168.10.0 0.0.0.255 REMOTE(config)#crypto map VPN_MAP 10 ipsec-isakmp REMOTE(config-crypto-map)#set peer 100.100.100.9 REMOTE(config-crypto-map)#set transform-set VPN_SET REMOTE(config-crypto-map)#match address 170 REMOTE(config-crypto-map)#ex REMOTE(config)#int s0/1/0 REMOTE(config-if)#crypto map VPN_MAP *Jan 3 07:16:26.785: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON | HQ#conf t HQ(config)#crypto isakmp policy 10 HQ(config-isakmp)#encr aes HQ(config-isakmp)#hash sha HQ(config-isakmp)#authentication pre-share HQ(config-isakmp)#group 2 HQ(config-isakmp)#lifetime 86400 HQ(config)#crypto isakmp key VPN_KEY address 100.100.100.14 HQ(config)#crypto ipsec transform-set VPN_SET esp-aes esp-sha-hmac HQ(config)#crypto map VPN_MAP 10 ipsec-isakmp HQ(config-crypto-map)#set peer 100.100.100.14 HQ(config-crypto-map)#set transform-set VPN_SET HQ(config-crypto-map)#match address 170 HQ(config-crypto-map)#ex HQ(config)#access-list 170 permit ip 192.168.10.0 0.0.0.255 192.168.10.136 0.0.0.7 HQ(config)#int s0/1/1 HQ(config-if)#crypto map VPN_MAP *Jan 3 07:16:26.785: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON |

CONFIGURATION:

Then the statue is active

AND

We can see encrypted packets:

How vpn commands works? And What are those?

REMOTE(config)#crypto isakmp policy 10                      # defines an ISAKMP policy with priority 10

REMOTE(config-isakmp)#encr aes                                   # it says encryption will be AES encryption for securing the VPN

REMOTE(config-isakmp)#hash sha                                  # it says that hashing will be SHA hashing for integrity verification

REMOTE(config-isakmp)#authentication pre-share      #pre-shared keys will be used for authentication

REMOTE(config-isakmp)#group 2                                    # VPN will use Diffie-Hellman Group 2 for key exchange

REMOTE(config-isakmp)#lifetime 86400                        # we set the lifetime(duration) for 24 hours

---IPsec configuration---

REMOTE(config)#crypto isakmp key VPN_KEY address 100.100.100.9                                          #pre-shared key for that HQ

REMOTE(config)#crypto ipsec transform-set VPN_SET esp-aes esp-sha-hmac                             # we will use esp-aes for encryption and esp-sha-hmac for hashing

REMOTE(config)#access-list 170 permit ip 192.168.10.136 0.0.0.7 192.168.10.0 0.0.0.255      #it says HQ router and Remote Network traffic will be encrypted

REMOTE(config)#crypto map VPN_MAP 10 ipsec-isakmp                                                                #Creates a crypto map for IPsec

REMOTE(config-crypto-map)#set peer 100.100.100.9                                                                      #We indicate the HQ router as a peer

REMOTE(config-crypto-map)#set transform-set VPN_SET                                                                #That defines encryption methods we set before

REMOTE(config-crypto-map)#match address 170                                                                              # The traffic encryption will be according to access-list 170

REMOTE(config-crypto-map)#ex                                                                                                            #exiting from crypto-map

REMOTE(config)#int s0/1/0                                                                                                                    #going to interface s0/1/0

REMOTE(config-if)#crypto map VPN_MAP                                                                                          #appling the crypto map that interface

*Jan 3 07:16:26.785: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON                   *Our VPN is active

HQ(config)#license boot module c1900 technology-package securityk9

#booted that security packet because without it we couldn’t  use VPN

SUM:

- At the beginning I wasn’t able to use VPN but instead of VPN I could just make TUNNEL. But that time it wouldn’t provide me encryption. So  I ended up loading and operating security packet like (k9).Then I created VPN_MAP which determines which tarffic will be encypted. Moreover, In encryption there was 2 phase:
-first one is ISAKMP policy , I indicated the hashing and encryption function such as aes and sha for tunnel security and pre-shared authentication.

-second one is Transform Set, in this I named set such as VPN_SET and defined hash and encryption function like esp-aes esp-sha-hmac for protecting actual data.

ETHERCHANNEL

| CORE | DATA |
| --- | --- |
| core#conf t core(config)#int range f0/8-10 range)#switchport mode trunk core(config-if-range)#sw tr nat vlan 99 core(config-if-range)#channel-group 1 mode on | DATA#conf t DATA(config)#int range f0/8-10 DATA(config-if-range)#sw tr nat vlan 99 DATA(config-if-range)#channel-group 1 mode on |

P = Port is bundled and working
SU = Layer 2, up and active

What are etherchannel commands?

core#conf t                                                                   #configure the terminal

core(config)#int range f0/8-10                                 # configure multi-interface at the same time

core(config-if-range)#switchport mode trunk        #if I making them trunk then that means they will be carrying more than one access interface

core(config-if-range)#sw tr nat vlan 99

#it is “switchport trunk native vlan 99” meaning native vlan for those interface is vlan 99. Native vlan carries untagged traffic.

core(config-if-range)#channel-group 1 mode on

# we created ether  channel to make multiple links act as one and help bandwith.

#Also it says we are naming etherchannel as 10

“Mode on “  is  I am configuring etherchannel without negotiation such as (LACP, PAgP)

SUM:

- There is  data center and generally traffic goes that way more, so I added ether channel there to reduce conjunction and increase bandwith. So I choose 3 interface and merge them as a one, after that made channel-group on them. Also, I made same configuration both sides. So now, the system will provide us efficiency

STP(Spanning-tree-protocol)

CORE                                                                                                               BR_CORE

We created that to prevent a broadcast storm and control the all traffic and of course for preventing looping

What commands are used?

core (config)# spanning-tree mode pvst(                   #making its mode pvst to run STP separately for each VLAN

core (config)# spanning-tree vlan 10                           #priority 0 like image we decided give the highest priority to that vlan

data(config)#interface range f0/1-4                              #going to a range(f0/1-4)
data(config-if-range )#spanning-tree portfast              #make their connection fast

*I do this for all access interface and that provides me when I open the pkt.file  at the begging of everything we see some orange (disconnection) but when the access interfaces configured as portfast, then they won’t wait for negotiation and will be able to send packets. We couldn’t do it for trunks because it is not possible.

THE CHELLANGE I FACED

|  | After configuration like beside, T defines Trunk ports A defines Access ports |
| --- | --- |

Problem_1:  I should have created vlans on each sw before determining as a client because I didn’t and get the error of (‘VTP VLAN configuration not allowed when device is in CLIENT mode.’)

Problem_1.2:   If I made configuration right that kind of error wouldn’t pop up, because we are already using VTP not to make switch configure a vlan on its own.
Problem_2:  When I created vtp ı put all switch same vtp domain network but to give different password for each switch made vtp run out. Hence, I should have given same password to run VTP correctly.



Problem_3: 
When I write a rule for dns
there was a problem the destination port was always changing so I write something like this

Extended IP access list 130

10 permit tcp host 192.168.5.2 host 192.168.10.6 eq ftp

20 permit tcp host 192.168.5.34 host 192.168.10.6 eq ftp

30 permit tcp host 192.168.5.66 host 192.168.10.6 eq ftp

40 deny tcp any host 192.168.10.6 eq ftp

50 permit tcp 192.168.5.0 0.0.0.255 host 192.168.10.7 eq smtp

60 permit udp host 192.168.1.2 any eq domain

65 permit udp any host 192.168.1.2 eq domain

70 permit ip any host 192.168.1.2

80 permit tcp 192.168.5.0 0.0.0.255 host 192.168.10.5 eq 443 (10 match(es))

90 permit eigrp any any (313 match(es))

100 deny icmp any 192.168.10.0 0.0.0.31 (4 match(es))

110 permit icmp any any (85 match(es))

HQ(config-ext-nacl)#no 60

HQ(config-ext-nacl)#no 65

HQ(config-ext-nacl)#60 permit udp host 192.168.1.2 any

HQ(config-ext-nacl)#no 70

HQ(config-ext-nacl)#70 permit udp any host 192.168.1.2 eq 53

--Solution--

Then I solve that problem the packet come from ISP_DHCP woudn’t be configured like  60 permit udp host 192.168.1.2 any eq domain
because we don’t know port number the destination is 53 but source is random one so when we write acl which deny any ip to reach on s0/1/1, it deny every packet even though we write acl to prevent this I couldn’t notice that when ISP_DHCP answer the packet but it will put the random port as a destination. Therefore, that ACL wouldn’t work because the rule is not related to random port.

So I applied that:

60 permit udp host 192.168.1.2 any                                  I wrote that permission without “eq domain” so can make ISP_DHCP sent its answer.

Problem_3:   I tried both side nat conf but when ı do that hq and branch start not to ping
Then I had got this scnerio in simulation and I saw everything was okay but when the packed come back they was changing source address, So the first destination address and expected address wasn’t watching. Because of this, I couldn’t ping hq_vlans to branch_vlans
Then I thought there would be way and decided to use extended acl 
Solution:
I learned when we want two network which has NAT connected , we cannot connect without writing specific ACL,SO I wrote ACL 110 for that reason. And it says when a packet want to go BRANCH : don’t let IP converted when the packet goes to HQ.

I used that:

Extended IP access list 110

10 deny ip 192.168.10.0 0.0.0.127 192.168.5.0 0.0.0.127

20 permit ip 192.168.10.0 0.0.0.127 any

Problem 4: VPN creating 
Solution:
I handle it with loading a packet. So, it works on remote side.
I got the license.

HQ(config)#license boot module c1900 technology-package securityk9

PROBLEM 5: IPV6 and NAT/64

No solution due to packet tracer

I couldn’t make nat-pt/nat64 conf because packet tracer doesn’t support it, even so I have given hq s0/1/1 interface ipv6 IP address but it wont be used.